Copyright 2018 AIA
MasterSpec Full Length
03/18 (PM updated 04/22)

PRODUCT MASTERSPEC LICENSED BY DELTEK, INC. TO KOLBE WINDOWS & DOORS

Copyright 2018 by The American Institute of Architects (AIA)

Exclusively published and distributed by Deltek, Inc. for the AIA

This Product MasterSpec Section is licensed by Deltek, Inc. to Kolbe Windows & Doors ("Licensee").

This Product MasterSpec Section modifies the original MasterSpec text and does not include the full content of the original MasterSpec Section.

Revisions made to the original MasterSpec text are made solely by the Licensee and are not endorsed by, or representative of the opinions of, Deltek or The American Institute of Architects (AIA). Neither AIA nor Deltek are liable in any way for such revisions or for the use of this Product MasterSpec Section by any end user. A qualified design professional should review and edit the document to suit project requirements.

For more information, contact Kolbe Windows & Doors, 1323 S. 11th Ave., Wausau, WI 54401; Phone: (715) 842-5666; Website: www.kolbewindows.com; Email: professionals@kolbewindows.com.

For information about MasterSpec, contact Deltek at (800) 424-5080 or visit masterspec.com.
SECTION 085200 - WOOD WINDOWS
TIPS:
To view non-printing Editor's Notes that provide guidance for editing, click on MasterWorks/Single-File Formatting/Toggle/Editor's Notes.
To read detailed research, technical information about products and materials, and coordination checklists, click on MasterWorks/Supporting Information.

Access Product MasterSpec Sections:

<Double click here to view the list of manufacturer Sections available at ProductMasterSpec.com>
Access BIM Content on BIMsmith Market:

<Double click here to view products on BIMsmith Market>
Revise this Section by deleting and inserting text to meet Project-specific requirements.MasterSpec includes provisions for LEED v4, LEED v4.1, IgCC, and Green Globes.
PART 1 -  GENERAL

1.1 SUMMARY

Retain applicable option in paragraph below for clad wood windows, or delete options if windows are not clad.

A. Section Includes:

1. Aluminum-clad wood windows.

2. Wood windows.

1.2 PREINSTALLATION MEETINGS

Retain "Preinstallation Conference" Paragraph below if Work of this Section is extensive or complex enough to justify a conference.

A. Preinstallation Conference: Conduct conference at [Project site] <Insert location>.

Retain subparagraphs below if required. If retaining, revise to include product-specific requirements.

1. Review and finalize construction schedule and verify availability of materials, Installer's personnel, equipment, and facilities needed to make progress and avoid delays.

2. Review, discuss, and coordinate the interrelationship of wood windows with other exterior wall components. Include provisions for anchoring, flashing, weeping, sealing perimeters, and protecting finishes.

3. Review and discuss the sequence of work required to construct a watertight and weathertight exterior building envelope.

4. Inspect and discuss the condition of substrate and other preparatory work performed by other trades.

If needed, insert list of conference participants not mentioned in Section 013100 "Project Management and Coordination."

1.3 ACTION SUBMITTALS
Action submittals are submittals requiring responsive action and return of reviewed documents to Contractor.
A. Product Data: For each type of product.

1. Include construction details, material descriptions, glazing and fabrication methods, dimensions of individual components and profiles, hardware, and finishes for wood windows.
Retain "Sustainable Design Submittals" Paragraph below if required to attain sustainability rating or to track sustainability submittals.
B. Sustainable Design Submittals:

For Kolbe's FSC certificate, see https://info.fsc.org/details.php?id=a0240000005sVLmAAM&type=certificate.

1. Chain-of-Custody Certificates: For certified wood products. Include statement of costs.
"Chain-of-Custody Qualification Data" Subparagraph below applies to LEED v4 and LEED v4.1.

2. Chain-of-Custody Qualification Data: For manufacturer and vendor.
C. Shop Drawings: For wood windows.

1. Include plans, elevations, sections, hardware, accessories, insect screens, operational clearances, and details of installation, including anchor, flashing, and sealant installation.

Retain "Samples" Paragraph below for single-stage Samples, with a subordinate list if applicable. Retain "Samples for Initial Selection" and "Samples for Verification" paragraphs for two-stage Samples.

D. Samples: For each exposed product and for each color specified, [2 by 4 inches (50 by 100 mm)] <Insert dimensions> in size.

E. Samples for Initial Selection: For units with factory-applied finishes.

1. Include Samples of hardware and accessories involving color selection.

F. Samples for Verification: For wood windows and components required, prepared on Samples of size indicated below:

1. Exposed Finishes: [2 by 4 inches (50 by 100 mm)] <Insert dimensions>.

2. Exposed Hardware: Full-size units.

G. Product Schedule: For wood windows. Use same designations indicated on Drawings.

1.4 INFORMATIONAL SUBMITTALS

Informational submittals are submittals that require review by Architect, but they do not require Architect's responsive action and return of reviewed documents to Contractor, provided submittals comply with requirements. If rejected, submittals with responsive action must be returned to Contractor.
Coordinate "Qualification Data" Paragraph below with qualification requirements in Section 014000 "Quality Requirements" and as may be supplemented in "Quality Assurance" Article.

A. Qualification Data: For Installer.

B. Product Test Reports: For each type of wood window, for tests performed by a qualified testing agency.

Retain "Field quality-control reports" Paragraph below if Contractor is responsible for field quality-control testing and inspecting.

C. Field quality-control reports.

D. Sample Warranties: For manufacturer's warranties.

1.5 QUALITY ASSURANCE

Retain "Manufacturer Qualifications" Paragraph below if required for LEED v4 and LEED v4.1.

For Kolbe's FSC certificate, see https://info.fsc.org/details.php?id=a0240000005sVLmAAM&type=certificate.

A. Manufacturer Qualifications: A qualified manufacturer that is certified for chain of custody by an FSC-accredited certification body.
Retain "Vendor Qualifications" Paragraph below if required for LEED v4 and LEED v4.1.

B. Vendor Qualifications: A vendor that is certified for chain of custody by an FSC-accredited certification body.
C. Installer Qualifications: An installer acceptable to wood window manufacturer for installation of units required for this Project.

D. Mockups: Build mockups to verify selections made under Sample submittals, to demonstrate aesthetic effects, and to set quality standards for fabrication and installation.

Indicate portion of wall represented by mockup on Drawings or draw mockup as separate element.

1. Build mockup of typical wall area as indicated on Drawings.

2. Approval of mockups does not constitute approval of deviations from the Contract Documents contained in mockups unless Architect specifically approves such deviations in writing.

Retain subparagraph below if the intention is to make an exception to the default requirement in Section 014000 "Quality Requirements" for demolishing and removing mockups.

3. Subject to compliance with requirements, approved mockups may become part of the completed Work if undisturbed at time of Substantial Completion.

Insert "Field Conditions" Article and requirements for field measurements for existing construction that requires custom-fabricated window replacements.

1.6 WARRANTY

When warranties are required and available, verify with Owner's counsel that special warranties stated in this article are not less than remedies available to Owner under prevailing local laws.

A. Manufacturer's Warranty: Manufacturer agrees to repair or replace wood windows that fail in materials or workmanship within specified warranty period.

Verify available warranties and warranty periods for units and components..

1. Warranty Period:

a. Window: [10] <Insert number> years from date of shipment and issued to the owner at Substantial Completion.

Warranty period for glazing units varies by type.

b. Glazing Units: [Five] [10] [20] <Insert number> years from date of shipment and issued to the owner at Substantial Completion.

Warranty period for aluminum finish depends on type. Baked-on enamel finishes often have 10-year warranties; high-performance fluoropolymer finishes often have 20-year warranties.

c. Aluminum-Cladding Finish: [10] [20] [30] <Insert number> years from date of shipment and issued to the owner at Substantial Completion.

PART 2 -  PRODUCTS

Manufacturers and products listed in this Section are neither recommended nor endorsed by the AIA or Deltek. Before selecting manufacturers and products, verify availability, suitability for intended applications, and compliance with minimum performance requirements. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

Product options commonly available from manufacturers are included in square brackets throughout the Section Text. Not every manufacturer listed can provide every option offered; verify availability with manufacturers. For definitions of terms and requirements for Contractor's product selection, see Section 016000 "Product Requirements."

2.1 SOURCE LIMITATIONS
A. Obtain wood windows from single source from single manufacturer.

2.2 WINDOW PERFORMANCE REQUIREMENTS

See the Evaluations for discussions of performance requirements. Coordinate performance requirements with types of window operation (for example, casement or double hung), glass type, and other variables. If performance requirements vary among types of windows, insert language to differentiate requirements among window types or show requirements in a schedule on Drawings.

A. Product Standard: Comply with AAMA/WDMA/CSA 101/I.S.2/A440 for definitions and minimum standards of performance, materials, components, accessories, and fabrication unless more stringent requirements are indicated.

Retain requirements for WDMA certification and labeling if needed. Not all manufacturers that test products in accordance with AAMA/WDMA/CSA 101/I.S.2/A440 requirements participate in WDMA's third-party certification program for listing and labeling wood windows.

1. Window Certification: WDMA certified with label attached to each window.

B. Performance Class and Grade: AAMA/WDMA/CSA 101/I.S.2/A440 as follows:

For Kolbe's "Ultra Series," "VisaLuxe Collection," and "Heritage Series" doors performance class and grade, see www.kolbewindows.com/resources/professional-tools/performance-class-grade.
Stock wood windows are generally unavailable in Class AW.

1. Minimum Performance Class: [R] [LC] [CW] [As indicated on Drawings] <Insert class>.

AAMA/WDMA/CSA 101/I.S.2/A440 establishes a gateway Performance Grade for windows to qualify for each Performance Class. The gateway Performance Grade is 15 for Class R, 25 for Class LC, and 30 for Class CW. For a particular project, the minimum Performance Grade for windows is typically based on the design pressure.

2. Minimum Performance Grade: [15] [20] [25] [30] [35] [40] [45] [50] [As indicated on Drawings] <Insert grade>.

Retain "Thermal Transmittance" Paragraph below if required to comply with requirements of authorities having jurisdiction. Options below are based on ENERGY STAR requirements. First option is for Northern Climate Zone, second is for North-Central Climate Zone, third is for South-Central Climate Zone, and fourth is for Southern Climate Zone.
For Kolbe's "Ultra Series," "VisaLuxe Collection," and "Heritage Series" windows "Energy Performance," see www.kolbewindows.com/resources/professional-tools/energy-performance.
C. Thermal Transmittance: NFRC 100 maximum whole-window U-factor of [0.30 Btu/sq. ft. x h x deg F (1.71 W/sq. m x K)] [0.32 Btu/sq. ft. x h x deg F (1.83 W/sq. m x K)] [0.35 Btu/sq. ft. x h x deg F (2.0 W/sq. m x K)] [0.60 Btu/sq. ft. x h x deg F (3.43 W/sq. m x K)] <Insert value>.

Options in "Solar Heat-Gain Coefficient (SHGC)" Paragraph below are based on ENERGY STAR requirements. First option is for North-Central Climate Zone, second is for South-Central Climate Zone, and third is for Southern Climate Zone. Northern Climate Zone does not have a maximum SHGC.

D. Solar Heat-Gain Coefficient (SHGC): NFRC 200 maximum whole-window SHGC of [0.52] [0.47] [0.40] [0.30] [0.27] <Insert value>.

Retain "Sound Transmission Class (STC)" and "Outside-Inside Transmission Class (OITC)" paragraphs below after verifying availability of test data. STC evaluates construction subject to interior sound frequencies, while OITC evaluates an expanded sound-frequency range more representative of conditions to which the building envelope is subject, such as road, rail, and airplane traffic noise. OITC is generally the preferred evaluation method for exterior windows.
Kolbe's windows range between 26 and 41 STC and 21 and 36 OITC; for more information, see www.kolbewindows.com/resources/professional-tools/acoustic.
E. Sound Transmission Class (STC): Rated for not less than [26] [27] [28] [29] [30] [31] [32] [33] [34] [35] [36] [37] [38] [39] [40] [41] <Insert rating> STC when tested for laboratory sound transmission loss in accordance with ASTM E90 and determined by ASTM E413.

F. Outside-Inside Transmission Class (OITC): Rated for not less than [21] [22] [23] [24] [25] [26] [27] [28] [29] [30] [31] [32] [33] [34] [35] [36] <Insert rating> OITC when tested for laboratory sound transmission loss in accordance with ASTM E90 and determined by ASTM E1332.

Retain "Windborne-Debris Impact Resistance" Paragraph below to suit Project. The IBC establishes criteria for buildings in hurricane-prone locations. In paragraph, "enhanced" option applies to essential facilities and has additional requirements. Verify requirements of authorities having jurisdiction. Verify which manufacturers have tested products and can demonstrate compliance.
Kolbe's "Heritage Series" and "Ultra Series" have products capable of complying with IPD4 and IPD3; for more information, see www.kolbewindows.com/solutions/impact-resistance.
G. Windborne-Debris Impact Resistance: Passes ASTM E1886 missile-impact and cyclic-pressure tests in accordance with ASTM E1996 for Wind Zone [1] [2] [3] [4] for [basic] [enhanced] protection.

Insert increased heights if different from those in "Large-Missile Test" and "Small-Missile Test" subparagraphs below. For enhanced protection, delete "Small-Missile Test" Subparagraph.

1. Large-Missile Test: For glazing located within [30 ft. (9.1 m)] <Insert dimension> of grade.

2. Small-Missile Test: For glazing located between 30 ft. (9.1 m) and [60 ft. (18.3 m)] <Insert dimension> above grade.

2.3 WOOD WINDOWS

A. Aluminum-Clad Wood Windows:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed.
1. Basis-of-Design Product: Subject to compliance with requirements, provide Kolbe Windows & Doors; [Ultra Series] [VistaLuxe Collection] or comparable product by one of the following:
a. Loewen Windows.

b. Marvin Windows and Doors.

c. <Insert manufacturer's name>.
B. Wood Windows:

Retain "Basis-of-Design Product" Subparagraph and list of manufacturers below to identify a specific product or a comparable product from manufacturers listed.
1. Basis-of-Design Product: Subject to compliance with requirements, provide Kolbe Windows & Doors; Heritage Series or comparable product by one of the following:
a. Loewen Windows.
b. Marvin Windows and Doors.

c. <Insert manufacturer's name>.
C. Operating Types: Provide the following operating types in locations indicated on Drawings:

1. Casement: Project out.

2. Awning: Project out.

3. Single hung.

4. Double hung.

5. Horizontal sliding.
6. Multi-panel folding.

7. Fixed.

Kolbe offers a limited number of custom units; for more information, contact the manufacturer.
8. <Insert operating type>.
First "Certified Wood" Paragraph below applies to LEED v4 and LEED v4.1. Manufacturers certifying products as "FSC Mixed Credit" do not have to use 100 percent certified wood in the products; however, in their total production, manufacturers must use an amount equal to or greater than the percentage of their production that is labeled "FSC Mixed Credit."

For Kolbe's FSC certificate, see https://info.fsc.org/details.php?id=a0240000005sVLmAAM&type=certificate.

D. Certified Wood: Wood products are to be certified as "FSC Pure"[ or "FSC Mixed Credit"] in accordance with FSC STD-01-001 and FSC STD-40-004.

First "Certified Wood" Paragraph below applies to IgCC.

E. Certified Wood: Wood products are to be labeled in accordance with the AF&PA's Sustainable Forestry Initiative, be certified as "FSC Pure" in accordance with FSC STD-01-001 and FSC STD-40-004, or be certified and labeled in accordance with the standards of the Programme for Endorsement of Forest Certification.

First "Certified Wood" Paragraph below applies to ASHRAE 189.1.

F. Certified Wood: Wood products are to [contain not less than 60 percent] [be made from] certified wood tracked through a chain-of-custody process. Certified wood documentation is to be provided by sources certified through a forest certification system with principles, criteria, and standards developed using ISO/IEC Guide 59 or the World Trade Organization's "WTO Agreement on Technical Barriers to Trade."

"Certified Wood" Paragraph below applies to Green Globes.

G. Certified Wood: Wood products are to be certified in accordance with the American Tree Farm System's "AFF Standard," the AF&PA's Sustainable Forestry Initiative, FSC STD-01-001 and FSC STD-40-004, or the standards of the Programme for Endorsement of Forest Certification.
AAMA/WDMA/CSA 101/I.S.2/A440 includes requirements for wood, cladding materials, and other materials and window components. If more stringent requirements apply, insert them in this article.

H. Frames and Sashes: Fine-grained wood lumber complying with AAMA/WDMA/CSA 101/I.S.2/A440; kiln-dried to a moisture content of not more than 12 percent at time of fabrication; free of visible finger joints, blue stain, knots, pitch pockets, and surface checks larger than 1/32 inch (0.8 mm) deep by 2 inches (51 mm) wide; water-repellent preservative treated.

Retain "Exterior Aluminum Surfaces" Subparagraph below for Kolbe's "Ultra Series" and "Vistaluxe Collection" aluminum-clad wood-framed windows.
Retain first option below for Kolbe's "Ultra Series" or "VistaLuxe WD LINE" standard finish; retain second option for coastal and heavy salt concentration areas.

1. Exterior Aluminum Surfaces: Aluminum cladding with manufacturer's standard [manufacturer's standard silicon modified polyester (SMP) coating complying with AAMA 2604] [fluoropolymer two-coat system with fluoropolymer color topcoat containing not less than 70 percent PVDF resin by weight and complying with AAMA 2605] [anodized coating] [mica coating complying with AAMA 2605].
a. SMP or PVDF Color: [Abalone] [Alabaster] [Anchor Gray] [Basil] [Beige] [Bronze] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coal black] [Coastal storm] [Copper canyon] [Corbeau] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Midnight] [Mystic ivy] [Mudpie] [Nutmeg] [Natural cotton] [Onyx] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.
b. Anodized Color: [Black] [Champagne] [Clear] [Dark bronze].
c. Mica Color: [Cosmic gray] [Galaxy silver] [Night sky] [Silverstorm].
Retain "Exterior Wood Surfaces" Subparagraph below for Kolbe's "Heritage Series" unclad wood-framed windows.

2. Exterior Wood Surfaces: [Manufacturer's standard factory-applied color finish] [Manufacturer's standard factory-applied latex primer] [Manufacturer's standard factory-applied oil primer] [Unfinished].
Retain "Paint Color" Subparagraph below for factory-applied color finish or modify for RAL custom paint color.

a. Paint Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Camel] [Castlerock] [Chutney] [Cloud] [Cider] [Coastal storm] [Copper canyon] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Mudpie] [Mystic ivy] [Natural cotton] [Nutmeg] [Sahara] [Papaya] [Roma] [Rustic] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.
Retain "Wood Species" Subparagraph below for unfinished wood.
Kolbe's standard wood species is "Pine"; custom wood specific other than those listed may be requested.

b. Wood Species: [Pine] [Alder] [Bamboo] [Cherry] [Knotty alder] [Mahogany] [Maple] [Mixed grain fir] [Oak] [Sapele] [Spanish cedar] [Vertical grain fir] [Walnut] [White oak] <Insert species>.
3. Interior Wood Surfaces: [Unfinished] [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Manufacturer's standard clear coat] [Manufacturer's standard stain-and varnish-finish].

Kolbe's standard wood species is "Pine"; custom wood species other than that listed may be requested.

a. Wood Species: [Pine] [Alder] [Bamboo] [Cherry] [Knotty alder] [Mahogany] [Maple] [Mixed grain fir] [Oak] [Sapele] [Spanish cedar] [Vertical grain fir] [Walnut] [White oak] <Insert color>.
Retain "Wood Stain Color" Subparagraph below for color-coated or stain-and-varnish finish or modify for custom color.

b. Wood Stain Color: [Cherry] [Chestnut] [Coffee bean] [Double clear coat] [Ebony] [Library red] [Light oak] [Red wheat] [Spiced walnut] [Sunset oak] [Wheat] <Insert species>.
c. Paint Color: [Abalone] [Bright white] [Coal black] [Graystone] [Ivory tusk] [Misty gray] [Natural cotton] [Silk] [Ultra pure white] <Insert color>.
4. Nailing Fin: Manufacturer's standard nailing fins for securing frame to structure during installation to hold unit plumb, square and true.

Retain "Installation Clips" Subparagraph below to suit Project.

5. Installation Clips: Provide manufacturer’s standard clips of sufficient strength to withstand design pressure indicated.

Retain "Glass," "Windborne-Debris-Impact-Resistant Laminated Glass," "Insulating-Glass Units," or "Windborne-Debris-Impact-Resistant Insulating-Glass Units" Paragraph below to suit Project, or revise if other type of glazing unit is necessary.

I. Glass: Clear annealed glass, ASTM C1036, Type 1, Class 1, q3.

Codes require fully tempered or laminated labeled safety glass in areas subject to human impact.

1. Kind: Fully tempered [where indicated on Drawings] <Insert requirements>.

Retain "Windborne-Debris-Impact-Resistant Laminated Glass" Paragraph below with "Windborne-Debris-Impact Resistance" Paragraph in "Window Performance Requirements" Article.

J. Windborne-Debris-Impact-Resistant Laminated Glass: ASTM C1172 with two plies of float glass.

1. Float Glass: [Annealed] [Heat strengthened] [Fully tempered] [As required by performance requirements indicated].

2. Inner Ply: Clear.

3. Interlayer: [0.090 inch (2.29 mm)] [As required by performance requirements indicated] <Insert requirements>.

4. Outer Ply: [Clear] [Gray] [Bronze] [Green] <Insert tint>.

Retain first or second option in "Low-E Coating" Subparagraph below for Kolbe's "Solar Low-E," "Solar Gain Low-E," "Solar Control Low-E," or "Solar Advanced Control Low-E"; retain third option for "ThermaPlus Low-E"; retain fourth option for triple-pane windows.
5. Low-E Coating: [Pyrolytic on second surface] [Sputtered on second surface] [Sputtered on second or fourth surface] [Sputtered on second or fifth surface].

K. Insulating-Glass Units: ASTM E2190.

1. Glass: ASTM C1036, Type 1, Class 1, q3.

a. Tint: [Clear] [Gray] [Bronze] [Green] <Insert tint>.

b. Kind: Fully tempered [where indicated on Drawings] <Insert requirements>.

2. Lites: [Two] [Three].

3. Filling: Fill space between glass lites with [air] [argon].

Retain first or second option in "Low-E Coating" Subparagraph below for Kolbe's "Solar Low-E," "Solar Gain Low-E," "Solar Control Low-E," or "Solar Advanced Control Low-E"; retain third option for "ThermaPlus Low-E"; retain fourth option for triple-pane windows.
4. Low-E Coating: [Pyrolytic on second surface] [Sputtered on second surface] [Sputtered on second or fourth surface] [Sputtered on second or fifth surface].
Retain "Windborne-Debris-Impact-Resistant Insulating-Glass Units" Paragraph below with "Windborne-Debris-Impact Resistance" Paragraph in "Window Performance Requirements" Article.

L. Windborne-Debris-Impact-Resistant Insulating-Glass Units: ASTM E2190 with two lites and complying with impact-resistance requirements in "Window Performance Requirements" Article.

1. Exterior Lite: ASTM C1036, Type 1, Class 1, q3.

a. Tint: [Clear] [Gray] [Bronze] [Green] <Insert tint>.

b. Kind: [Heat strengthened] [Fully tempered].

2. Interior Lite: ASTM C1172 clear laminated glass with two plies of float glass.

a. Float Glass: [Annealed] [Heat strengthened] [Fully tempered] [As required by performance requirements indicated].

b. Interlayer Thickness: [0.090 inch (2.29 mm)] [As required by performance requirements indicated] <Insert requirements>.

3. Filling: Fill space between glass lites with [air] [argon].

Retain first or second option in "Low-E Coating" Subparagraph below for Kolbe's "Solar Low-E," "Solar Gain Low-E," "Solar Control Low-E," or "Solar Advanced Control Low-E"; retain third option for "ThermaPlus Low-E"; retain fourth option for triple-pane windows.
4. Low-E Coating: [Pyrolytic on second surface] [Sputtered on second surface] [Sputtered on second or fourth surface] [Sputtered on second or fifth surface].

"Glazing System" Paragraph below refers to the method by which the glazing unit (glass) is retained within the window sash or frame.

M. Glazing System: [Manufacturer's standard factory-glazing system that produces weathertight seal] <Insert glazing requirements>.

"Dual Glazing System" Subparagraph below refers to windows glazed with two separate lites. Verify availability with manufacturers.

1. Dual Glazing System:

a. Interior Lite: [Glass] <Insert type>.

b. Exterior Lite: [Glass] [Insulating-glass unit] <Insert type>.

Nonmagnetic stainless steel, Series 300, or superior corrosion-resistant-coated metal hardware may be required to meet specific customer or regional needs and for protection against corrosive environments, such as in urban, coastal, or industrial areas.

N. Hardware, General: Provide manufacturer's standard hardware fabricated from aluminum, stainless steel, carbon steel complying with AAMA 907, or other corrosion-resistant material compatible with adjacent materials; designed to smoothly operate, tightly close, and securely lock windows, and sized to accommodate sash weight and dimensions.

1. Exposed Hardware Color and Finish: [As indicated by manufacturer's designations] [Match Architect's sample] [As selected by Architect from manufacturer's full range] [Antique brass] [Antique nickel] [Beige] [Brass] [Bright brass] [Clay] [Dull brushed chrome] [Matte black] [Oil-rubbed] [Rustic umber] [Satin nickel] [Smoky gray] [Stainless steel] [White].

O. Projected Window Hardware:

1. Gear-Type Rotary Operators: Complying with AAMA 901 when tested in accordance with ASTM E405, Method A. Provide operators that function without requiring the removal of interior screens or using screen wickets.

a. Type and Style: [Match Architect's sample] [As selected by Architect from manufacturer's full range of types and styles] <Insert type and style>.

2. Hinges: [Manufacturer's standard type for sash weight and size indicated] [Stainless steel hinges with stainless steel-reinforced, sliding nylon shoes] <Insert description>.

3. Multi-Point Locking System: Provide single handle to operate positive-acting arms that pull sash into locked position with locking at multiple points for casements. Provide single handle or double handle to operate positive-acting arms that pull sash into locked position with locking at multiple points for awnings.

Retain one option in "Limit Devices" Subparagraph below if limit devices are required. They restrict sash opening for safety. Indicate locations of windows with limit devices on Drawings or in schedules, and coordinate with code requirements for windows for emergency escape and rescue.

4. Limit Devices: [Concealed friction adjustor, adjustable stay bar] [Concealed support arms with adjustable, limited, hold-open] <Insert type> limit devices designed to restrict sash opening.

a. Limit clear opening to [4 inches (100 mm)] <Insert dimension> for ventilation; with custodial key release.

5. Operator Stud Cover: Matching operator handle finish. Provide in locations where operator handle is removed for controlled access.

Retain "Pole Operators" Subparagraph below if Project includes manually operated windows more than 72 inches (1800 mm) above floor. Electric operators are also available; if needed, insert requirements.

6. Pole Operators: Tubular-shaped anodized aluminum; with rubber-capped lower end and standard push-pull hook at top to match hardware design; of sufficient length to operate window without reaching more than 60 inches (1500 mm) above floor; one pole operator and pole hanger per room that has operable windows more than 72 inches (1800 mm) above floor.

P. Hung Window Hardware:

1. Counterbalancing Mechanism: Complying with AAMA 902, concealed, of size and capacity to hold sash stationary at any open position.

2. Locks and Latches: Allow unobstructed movement of the sash across adjacent sash in direction indicated and operated from the inside only.[ Provide custodial locks.]

3. Tilt Hardware: Releasing tilt latch allows sash to pivot about horizontal axis to facilitate cleaning exterior surfaces from the interior.

Q. Horizontal-Sliding Window Hardware:

1. Sill Cap/Track: Fiberglass track with integral black color of dimensions and profile indicated; designed to comply with performance requirements indicated and to drain to the exterior.

2. Locks and Latches: Allow unobstructed movement of the sash across adjacent sash in direction indicated and operated from the inside only.[ Provide custodial locks.]

3. Roller Assemblies: Low-friction design.

R. Weather Stripping: Provide full-perimeter weather stripping for each operable sash unless otherwise indicated.

S. Fasteners: Noncorrosive and compatible with window members, trim, hardware, anchors, and other components.

Generally, retain "Exposed Fasteners" Subparagraph below. Revise if exposed fasteners are permitted.

1. Exposed Fasteners: Do not use exposed fasteners to greatest extent possible. For application of hardware, use fasteners that match finish hardware being fastened.

2.4 ACCESSORIES

Retain "True Divided Lites" or "Grilles (False Muntins)" Paragraph below if required for Project. Some manufacturers have a limited range of options for grilles; confirm availability.
Kolbe's "Heritage Series" offers true divided lites; however, manufacturer range may be limited; for more information, see www.kolbewindows.com/product-lines/heritage-series.
A. True Divided Lites: Provide glazing with true muntins, resulting in individually glazed lites, in pattern indicated.
1. Wood Species: To match adjacent finish of window sash.

2. Design: [Rectangular] [As indicated on Drawings] <Insert design>.

3. Bar Width: Not less than 1-1/8 inches (28 mm) wide.
4. Interior Bar Profile: [Beveled] [Ovolo].
Retain first option in "Exterior Finish" Subparagraph below for Kolbe's "Ultra Series" or "VistaLuxe WD LINE" standard finish; retain second option for coastal and heavy salt concentration areas.

5. Exterior Finish: [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Unfinished].
a. Exterior Paint Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coastal storm] [Copper canyon] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Lunar] [Maize] [Mediterranean] [Mudpie] [Mystic ivy] [Nutmeg] [Khaki] [Natural cotton] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

6. Interior Finish: [Unfinished] [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Manufacturer's standard clear coat] [Manufacturer's standard stain-and varnish-finish].
a. Interior Stain Color: [Cherry] [Chestnut] [Coffee bean] [Double clear coat] [Ebony] [Library red] [Light oak] [Red wheat] [Spiced walnut] [Sunset oak] [Unfinished pine] [Wheat].

b. Interior Paint Color: [Abalone] [Bright white] [Coal black] [Graystone] [Ivory tusk] [Misty gray] [Natural cotton] [Silk] [Ultra pure white].
B. Grilles (False Muntins): Provide grilles in designs indicated to each sash lite.

1. Type: Exterior and interior, adhered to glass [with] [without] bars between the glass.
2. Material: To match adjacent finish of window sash.

3. Design: [Rectangular] [Diamond] [As indicated on Drawings] <Insert design>.

4. Bar Width: Not less than [5/8 inch (16 mm)] [7/8 inch (22 mm)] [1-1/8 inches (28 mm)] [1-3/8 inches (35 mm)] [1-3/4 inches (44.5 mm)] [2-1/4 inches (28 mm)] [4-1/2 inches (114 mm)] wide.
Retain one or more of first three options in "Interior Bar Profile" Subparagraph below for Kolbe's "Heritage Series" or "Ultra Series"; retain third or forth option for Kolbe's "VistaLuxe Collection."
5. Interior Bar Profile: [Beveled] [Ovolo] [Square] [Triangular].
Retain "Exterior Aluminum Finish" or "Exterior Wood Finish" Subparagraph below.

6. Exterior Aluminum Finish: Aluminum cladding with manufacturer's standard [silicon modified polyester (SMP) coating complying with AAMA 2604] [fluoropolymer two-coat system with fluoropolymer color topcoat containing not less than 70 percent PVDF resin by weight and complying with AAMA 2605] [anodized coating] [mica coating complying with AAMA 2605].

a. SMP or PVDF Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Bronze] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coal black] [Coastal storm] [Copper canyon] [Corbeau] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Midnight] [Mystic ivy] [Mudpie] [Nutmeg] [Natural cotton] [Onyx] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

b. Anodized Color: [Black] [Champagne] [Clear] [Dark Bronze].

c. Mica Color: [Cosmic gray] [Galaxy silver] [Night sky] [Silverstorm].
7. Exterior Wood Finish: [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Unfinished].
a. Exterior Paint Color: [Abalone] [Alabaster] [Anchor gray] [Basil] [Beige] [Camel] [Castlerock] [Chutney] [Cider] [Cloud] [Coastal storm] [Copper canyon] [Emerald isle] [Gingersnap] [Green tea leaf] [Hartford green] [Khaki] [Lunar] [Maize] [Mediterranean] [Mudpie] [Mystic ivy] [Natural cotton] [Nutmeg] [Papaya] [Roma] [Rustic] [Sahara] [Sapphire] [Shadow] [Shale] [Slate] [Smokestack] [Spiced vinegar] [Steel gray] [Timberwolf] [Truffle] [Ultra pure white] [White] <Insert color>.

8. Interior Wood Finish: [Unfinished] [Manufacturer's standard factory-applied primer] [Manufacturer's standard factory-applied color finish] [Manufacturer's standard clear coat] [Manufacturer's standard stain-and varnish-finish].
a. Interior Stain Color: [Cherry] [Chestnut] [Coffee bean] [Double clear coat] [Ebony] [Library red] [Light oak] [Red wheat] [Spiced walnut] [Sunset oak] [Wheat] <Insert color>.

b. Interior Paint Color: [Abalone] [Bright white] [Coal black] [Graystone] [Ivory tusk] [Misty gray] [Natural cotton] [Silk] [Ultra pure white] <Insert color>.
C. Casing: Extruded aluminum [1-5/8-inch (41-mm) brickmould] [1-15/16-inch (49-mm) brickmould] [2-inch (51-mm) flat casing] [2-inch (51-mm) flat casing with backband] [3-1/2-inch (89-mm) flat casing] [3-1/2-inch (89-mm) flat casing with backband] [3-1/2-inch (89-mm) profiled brickmould] [beaded casing] [beaded casing with backband] [bull nose casing] [historic brickmould] [ogee casing] [pavilion brickmould] [stucco trim casing].

D. Casing: Exterior wood [bull nose casing] [prospect casing] [1-15/16-inch (49-mm) flat casing] [1-15/16-inch (49-mm) brickmould] [2-inch (51-mm) beaded casing] [1-15/16-inch (49-mm) flat casing with backband (1-piece)] [3-1/2-inch (89-mm) profiled brickmould] [3-1/2-inch (89-mm) beaded casing] [3-1/2-inch (89-mm) flat casing] [3-1/2-inch (89-mm) profiled brickmould with backband] [3-1/2-inch (89-mm) flat casing with backband] [4-inch (102-mm) flat casing] [5-inch (127-mm) flat casing].
2.5 INSECT SCREENS

A. General: Fabricate insect screens to integrate with window frame. Provide screen for each operable exterior sash. Screen wickets are not permitted.

1. Type and Location: [Full, inside for project-out] [Full, outside for double-hung] [Half, outside for single-hung] [Full, outside for sliding] [Half, outside for sliding] [Interior, retractable for casements, awnings, or double hung] [Exterior, retractable for sliding] sashes.

B. Aluminum Frames: Manufacturer's standard aluminum alloy complying with SMA 1004 or SMA 1201. Fabricate frames with mitered or coped joints or corner extrusions, concealed fasteners, and removable PVC spline/anchor concealing edge of frame.

1. Tubular Framing Sections and Cross Braces: Roll formed from aluminum sheet.

2. Finish for Interior Screens: [Baked-on organic coating in color selected by Architect from manufacturer's full range] [Wood veneer matching species and finish of adjacent sash] <Insert color>.

3. Finish for Exterior Screens: [Baked-on organic coating in color selected by Architect from manufacturer's full range] [Matching color and finish of cladding].

Retain "Glass-Fiber Mesh Fabric" or "Aluminum Wire Fabric" Paragraph below or insert another.

Retain first option in "Glass-Fiber Mesh Fabric" Paragraph for Kolbe's standard "BetterVue" screen; retain second option for "UltraVue."

C. Glass-Fiber Mesh Fabric: [18-by-16 (1.0-by-1.1-mm)] [18-by-18 (1.0-by-1.0-mm)] count per sq. in. (sq. mm) mesh of PVC-coated, glass-fiber threads; woven and fused to form a fabric mesh resistant to corrosion, shrinkage, stretch, impact damage, and weather deterioration. Comply with ASTM D3656/D3656M.

1. Mesh Color: Black.

D. Aluminum Wire Fabric: 18-by-16 (1.1-by-1.3-mm) mesh of 0.011-inch- (0.28-mm-) diameter, coated aluminum wire.

1. Wire-Fabric Finish: [Brite coat aluminum] [Charcoal].

2.6 FABRICATION

A. Fabricate wood windows in sizes indicated. Include a complete system for installing and anchoring windows.

B. Glaze wood windows in the factory.

C. Weather strip each operable sash to provide weathertight installation.

Retain "Mullions" Paragraph below if required for Project.

D. Mullions: Provide mullions and cover plates, matching window units, complete with anchors for support to structure and installation of window units. Allow for erection tolerances and provide for movement of window units due to thermal expansion and building deflections. Provide mullions and cover plates capable of withstanding design wind loads of window units.

Bow and bay window assemblies usually contain casement, double-hung, or fixed window units.

E. [Bow] [Bay] Window Assemblies: Provide [operating] [and] [fixed] units in configuration indicated. Provide window frames, sashes, hardware, and other trim and components necessary for a complete, secure, and weathertight installation, including the following:

1. Angled mullion posts with interior and exterior trim.

2. Angled interior and exterior extension and trim.

Revise first subparagraph below to suit Project. Clear pine head and seat boards are optional features with most manufacturers. Unless wood species is specified, plywood will be supplied.

3. Clear [pine] [oak] head and seat boards.

4. Top and bottom plywood platforms.

5. Exterior head and sill casings and trim.

6. Support brackets.

F. Complete fabrication, assembly, finishing, hardware application, and other work in the factory to greatest extent possible. Disassemble components only as necessary for shipment and installation. Allow for scribing, trimming, and fitting at Project site.

2.7 WOOD FINISHES

Delete this article if finished, aluminum-clad wood-framed windows without primed interior surfaces or unfinished wood-framed windows are required.

A. Factory-Applied Primer: Provide manufacturer's standard factory-applied prime coat complying with WDMA T.M. 11 <Insert specific requirements for factory-applied prime coat if any> on exposed [exterior] [and] [interior] wood surfaces.

First option in "Factory-Applied Color Finish" Paragraph below is for pigmented (paint) coatings suitable for exterior exposure. Retain this paragraph if Andersen vinyl-clad hinged wood-framed glass doors are used because that product uses a high-performance urethane coating on the exterior of door panels.

B. Factory-Applied Color Finish: Provide manufacturer's standard factory-applied finish [complying with WDMA T.M. 12] <Insert manufacturer's standard finish>. 

2.8 ALUMINUM FINISHES

If retaining more than one finish in paragraphs below, indicate location of each on Drawings or by inserts.

A. Silicon Modified Polyester (SMP): AAMA 2604; epoxy primer and silicone-modified, polyester-enamel topcoat; with a dry film thickness of not less than 0.2 mil (0.005 mm) for primer and 0.8 mil (0.02 mm) for topcoat.

Retain "Basis-of-Design Product" Subparagraph below to identify a specific product or a comparable product.
For Kolbe's "Vantage Exterior SMP Finish" catalogue, see www.kolbewindows.com/resources/brochures.

1. Basis-of-Design Product: Subject to compliance with requirements, provide Kolbe Windows & Doors; Vantage Exterior SMP Finish or comparable product.
B. High-Performance Organic Finish: Two-coat fluoropolymer finish complying with AAMA 2605 and containing not less than 70 percent PVDF resin by weight in color coat. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.
C. Mica Fluoropolymer: AAMA 2605. Two-coat fluoropolymer finish with suspended mica flakes containing not less than 70 percent PVDF resin by weight in color coat. Prepare, pretreat, and apply coating to exposed metal surfaces to comply with coating and resin manufacturers' written instructions.

D. Color Anodic Finish: AAMA 611, [AA-M12C22A42/A44, Class I, 0.018 mm] [AA-M12C22A32/A34, Class II, 0.010 mm] or thicker.
PART 3 -  EXECUTION

3.1 EXAMINATION

A. Examine openings, substrates, structural support, anchorage, and conditions, with Installer present, for compliance with requirements for installation tolerances and other conditions affecting performance of the Work.

B. Verify rough opening dimensions, levelness of sill plate, and operational clearances.

C. Examine wall flashings, vapor retarders, water and weather barriers, and other built-in components to ensure weathertight window installation.

D. Proceed with installation only after unsatisfactory conditions have been corrected.

3.2 INSTALLATION

A. Comply with manufacturer's written instructions for installing windows, hardware, accessories, and other components. For installation procedures and requirements not addressed in manufacturer's written instructions, comply with installation requirements in ASTM E2112.

B. Install windows level, plumb, square, true to line, without distortion, anchored securely in place to structural support, and in proper relation to wall flashing and other adjacent construction to produce weathertight construction.

C. Separate aluminum and other corrodible surfaces from sources of corrosion or electrolytic action (dissimilar materials, treated lumber, and so on) at points of contact with other materials.

D. For fin method of attachment, integrate window system installation with exterior water-resistant barrier using flashing/sealant tape. Apply and integrate flashing/sealant tape with water-resistant barrier using watershed principles in accordance with window manufacturer's written instructions.

E. Place interior seal around window perimeter to maintain continuity of building thermal and air barrier using insulating-foam sealant.

F. Seal window to exterior wall cladding with sealant and related backing materials at perimeter of assembly.

G. Leave windows closed and locked.

3.3 FIELD QUALITY CONTROL

Retain "Testing Agency" Paragraph below to identify who will perform tests and inspections. If retaining second option in paragraph, retain "Field quality-control reports" Paragraph in "Informational Submittals" Article.

A. Testing Agency: [Owner will engage] [Engage] a qualified testing agency to perform tests and inspections.

1. Testing and inspecting agency will interpret tests and state in each report whether tested work complies with or deviates from requirements.

Revise "Testing Services" Paragraph below if more stringent testing is required.

B. Testing Services: Testing and inspecting of installed windows is to take place as follows:

1. Testing Methodology: Testing of windows for air infiltration and water resistance is to be performed in accordance with AAMA 502.

2. Air-Infiltration Testing:

a. Test Pressure: That required to determine compliance with AAMA/WDMA/CSA 101/I.S.2/A440 performance class indicated.

b. Allowable Air-Leakage Rate: [1.5] <Insert number> times the applicable AAMA/WDMA/CSA 101/I.S.2/A440 rate for product type and performance class rounded down to one decimal place.

3. Water-Resistance Testing:

a. Test Pressure: [Two-thirds] <Insert number> times test pressure required to determine compliance with AAMA/WDMA/CSA 101/I.S.2/A440 performance grade indicated.

b. Allowable Water Infiltration: No water penetration through the unit.

4. Testing Extent: [Three] [Three mockup] <Insert number or description> windows of each type as selected by Architect and a qualified independent testing and inspecting agency. Windows are to be tested after perimeter sealants have cured.

5. Test Reports: Prepared in accordance with AAMA 502.

See Section 014000 "Quality Requirements" for retesting and reinspecting requirements and Section 017300 "Execution" for requirements for correcting the Work.

C. Windows will be considered defective if they do not pass tests and inspections.

D. Prepare test and inspection reports.

3.4 ADJUSTING, CLEANING, AND PROTECTION

A. Adjust operating sashes and hardware for a tight fit at contact points and weather stripping for smooth operation and weathertight closure.

B. Clean exposed surfaces immediately after installing windows. Remove excess sealants, glazing materials, dirt, and other substances.

1. Keep protective films and coverings in place until final cleaning.

C. Remove and replace sashes if glass has been broken, chipped, cracked, abraded, or damaged during construction period.

D. Protect window surfaces from contact with contaminating substances resulting from construction operations. If contaminating substances do contact window surfaces, remove contaminants immediately in accordance with manufacturer's written instructions.

END OF SECTION 085200
	WOOD WINDOWS
	085200 - 6



